Welcome to the Small Radio Telescope (SRT)

The SRT is a great resource for high schools and colleges interested in having their students conduct real scientific experiments.  It is a versatile and capable instrument.  Technical information, software downloads and some lesson plans can be found at http://web.haystack.mit.edu/SRT/index.html

In the following 8 lessons, students learn how to operate the telescope, record and analyze data – all the skills they need to conduct their own investigations.  Here, the bare bones of the SRT are shown.  Follow the instructions on http://web.haystack.mit.edu/SRT/SRTexe.html to install your software.

Open up My Computer   C:\ drive    SRT folder find the SRTcatalog file that looks like the following

Right-click on this file and open it with Notepad

you should see the following code.

Pay close attention to the lines:

* SIMULATE RECEIVER

and

* SIMULATE ANTENNA 20

The asterisks in front of these lines indicate that they are not read by the computer.  If, for some reason, either the receiver or antenna mount are not working, this computer program can simulate their function.  To do this, simply delete the asterisks before these lines.  The “20” indicates that the simulated antenna will move 20 times faster than the real antenna.  It is useful to simulate the antenna in order to have it test command files.  More on that later.